ENGINEERING AND ENVIRONMENTAL SERVICES

Stormwater Division

120 East Parrish Street, 1st Floor, Durham, NC 27701

Telephone (919) 560-0739 Fax (919) 560-0740

BIORETENTION AREA DESIGN SUMMARY

Stormwater Management Construction Plan Review:

A complete Stormwater management construction plan submittal includes a design summary for each Stormwater BMP, design calculations, plans and specifications showing BMP, inlet and outlet structure details.

- PROJECT INFORMATION

For projects with multiple SCMs, specify which SCM this worksheet applies to:

Project Name: ___________________________________	Phase: _______________________

PIN: ___________________________________________	Case #: _______________________

Legal Name of Owner: ___________________________________________________________

Owner Contact: _________________________________	Phone: _______________________

Owner Address: ________________________________________________________________

Design Contact Person: ___________________________	Phone: _______________________

| Drainage area | ac |
| --- | --- |
| Impervious area | ac |
| Design runoff | cfs |
| Design treatment volume | ft3 |

(on-site drainage to the SCM)

(on-site impervious area draining to the SCM)

- GENERAL MINIMUM DESIGN CRITERIA FOR ALL SCMs

GENERAL MDC 1: SIZING

| Design storm depth |  | ft |
| --- | --- | --- |
| Design runoff volume |  | ft3 |

(One year, 24 hour storm event)

(Minimum calculation of entire drainage area)
GENERAL MDC 2: CONTAMINATED SOILS

| Contaminated soils within footprint? | Y / N |
| --- | --- |

(Brownfield location?)

GENERAL MDC 3: SIDE SLOPES

| Maximum vegetated side slopes | : 1 |
| --- | --- |

(Maximum 3:1 vegetated slopes)

GENERAL MDC 4: EROSION PROTECTION

| 10 year storm outlet discharge | cfs |
| --- | --- |

(Must be non-erosive)

GENERAL MDC 5: EXCESS FLOW

| Emergency outlet elevation |  | ft |
| --- | --- | --- |
| Emergency spillway width |  | ft |
| Emergency spillway side slopes |  | : 1 |
| Emergency spillway slope |  | % |

GENERAL MDC 6: DEWATERING

| Dewatering method |  |  |
| --- | --- | --- |
| Drawdown orifice size |  | in |

(If applicable)

GENERAL MDC 7: MAINTENANCE ACCESS

| Maintenance access width |  | ft |
| --- | --- | --- |
| Side slopes within maintenance access |  | : 1 |
| Access extend to public right of way |  | Y / N |

(Minimum width of 10 feet)

(Maximum 3:1)

- BIORETENTION MINIMUM DESIGN CRITERIA

BIORETENTION MDC 1: SEPARATION FROM THE SHWT

| SHWT elevation |  | ft |
| --- | --- | --- |
| Bottom elevation of media |  | ft |
| SHWT separation |  | ft |

(Determined through soil tests)

(SHWT elevation minus bottom elev)

BIORETENTION MDC 2: MAXIMUM PONDING DEPTH FOR DESIGN VOLUME

| Planting soil top elevation |  | ft |
| --- | --- | --- |
| Design volume ponding elevation |  | ft |
| Design volume ponding depth |  | in |
| Required treatment volume |  | ft3 |
| Required surface area |  | ft2 |

(Elevation of top of soil)

(Maximum elevation of design storm)

(Maximum 12 inches)

(Required volume divided by ponding depth)

BIORETENTION MDC 3: PEAK ATTENUATION VOLUME

| Peak attenuation volume depth |  | in |
| --- | --- | --- |
| Peak attenuation outlet elevation |  | ft |

(Max. 24” above planting surface elev.)

(Max. 18” above planting surface elev.)

BIORETENTION MDC 4: UNDERDRAIN

| Internal Water Storage included? |  | Y / N |
| --- | --- | --- |
| Top elevation of IWS |  | ft |
| IWS and planting surface separation |  | in |
| In-situ soil infiltration rate |  | in/hr |

(Min. 18 inches below planting surface)

(Min. 18 inches)

(Min. 2 in/hr to exclude underdrain)

BIORETENTION MDC 5: MEDIA DEPTH

| Does cell have shrubs or trees? |  | Y / N |
| --- | --- | --- |
| Media depth |  | ft |
| IWS and planting surface separation |  | ft |

(Min. depth 36” with trees and shrubs)

(Min. depth 24” with no IWS, 30” with IWS and no trees and shrubs)

BIORETENTION MDC 6: MEDIA MIX

| Medium to coarse washed sand |  | % |
| --- | --- | --- |
| Fines (silt and clay) |  | % |
| Organic matter |  | % |
| Volume of total media |  | ft3 |

(75 to 85 percent required)

(8 to 15 percent required)

(5 to 10 percent required)

BIORETENTION MDC 7: MEDIA P-INDEX

| Phosphorous Index |  |
| --- | --- |

(Less than 30 in NSW waters, less than 50 elsewhere)

- BIORETENTION DESIGN CHARACTERISTICS

Size of cell area			__________ ft2

Inflow sheet flow velocity 			__________ fps

Mulch layer top elevation			__________ ft

Media top elevation			__________ ft

Bottom elevation of cell			__________ ft

Perforated Pipe length			__________ ft

Space between pipe runs			__________ ft

Longitudinal slope			__________ ft/ft

Diameter of pipe			__________ ft

Emergency Spillway Information

Emergency Outlet Elev. 	__________ ft (invert of emergency overflow weir)

Emergency Spillway 	width _____ ft	side slopes ___:1	slope ______%

- REQUIRED ITEMS CHECKLIST

The following checklist outlines design requirements. Initial in the space provided to indicate the following design requirements have been met and supporting documentation is attached.

Applicant’s Initials

__________ a. Runoff from landscaped areas and other non-impervious areas has been                      directed away from the bioretention area to the maximum extent practical.

__________ b. Plan specifies how all slopes draining to the bioretention area will be stabilized. Note that the slopes must be stabilized before installation of the under drain system.

__________ c. Construction sequencing shall be considered and a note added to the plan that states: “All sediment and erosion control practices shall be in place and the slopes draining to the bioretention area shall be stabilized before construction of the bioretention area begins.”

__________ d. No side slopes draining to bioretention area greater than 3:1 (promote sheet flow through the grass filter strip.)

__________ e. Riprap outlet protection, if provided, reduces flow to non-erosive velocities (provide calculations).

__________ f. A recorded drainage easement is provided for each cell including access to the nearest right-of-way and is graded per Section 8.3, Stormwater Control Facilities.

__________ g. A plan view of the bioretention area with grading shown is provided.

__________ h. A profile through the bioretention area and emergency spillway is provided.

__________ i. Geotextile fabric is placed at the bottom of the excavated cell to prevent soil from migrating into the underdrain system.

__________ j. The underdrain system is wrapped in a gravel jacket and a geotextile fabric is placed between the bottom of the sand bed or planting soil and the top of the gravel jacket.

__________ k. The pipe for the underdrain system shall be perforated Schedule 40 PVC.

__________ l. The underdrain system shall connect to the outflow system at a point at least 1 foot inside the bioretention cell wall.

__________ m. A non-perforated Sch. 40 PVC piping system is connected to the underdrain piping and extends to the surface of the planting soil for cleanouts with screw on cleanout.

__________ n. Cleanouts are to be provided at the beginning of each pipe run and at all intersections.

__________ o. A planting soil mixture specification and a soil characteristics table are provided.

__________ p. The hydraulic conductivity for the planting mixture is to be 1-inch to 2-inches per hour.

__________ q. Mulch layer specification is provided; mulch is to be a 2 to 4-inch layer of triple shredded mulch with re-application rate specified.

__________ r. Soil with a P index of less than or equal to 30 in NSW waters, less than 50 elsewhere, specified on plan.

__________ s. A bioretention area landscape plan is provided including the preparation of the planting pit, fertilization, installation of the plant material, type and number of plantings (note that there shall be a minimum of three species of trees and three species of shrubs selected to ensure diversity, their planting locations, Operation and Maintenance checklist for owner.

__________ t. Note on plans: “The media shall not be mechanically compacted.”

NOTE: Executed Stormwater Facility Operations and Maintenance Permit Agreement and payment of surety are required prior to Stormwater Permit issuance.