ENGINEERING AND ENVIRONMENTAL SERVICES

Stormwater Division

120 East Parrish Street, 1st Floor, Durham, NC 27701

Telephone (919) 560-0739 Fax (919) 560-0740

SAND FILTER DESIGN SUMMARY

Stormwater Management Construction Plan Review:

A complete Stormwater management construction plan submittal includes a design summary for each Stormwater BMP, design calculations, plans and specifications showing BMP, inlet and outlet structure details.

- PROJECT INFORMATION

For projects with multiple SCMs, specify which SCM this worksheet applies to:

Project Name: ___________________________________	Phase: _______________________

PIN: ___________________________________________	Case #: _______________________

Legal Name of Owner: ___________________________________________________________

Owner Contact: _________________________________	Phone: _______________________

Owner Address: ________________________________________________________________

Design Contact Person: ___________________________	Phone: _______________________

Detention provided for:  ____ 1-year     _____ 2-year     _____ 10-year     _____other_________

- GENERAL MINIMUM DESIGN CRITERIA FOR ALL SCMs

GENERAL MDC 1: SIZING

| Design storm depth |  | ft |
| --- | --- | --- |
| Design runoff volume |  | ft3 |

(One year, 24 hour storm event)

(Minimum calculation of entire drainage area)
GENERAL MDC 2: CONTAMINATED SOILS

| Contaminated soils within footprint? | Y / N |
| --- | --- |

(Brownfield location?)

GENERAL MDC 3: SIDE SLOPES

| Maximum vegetated side slopes | : 1 |
| --- | --- |

(Maximum 3:1 vegetated slopes)

GENERAL MDC 4: EROSION PROTECTION

| 10 year storm outlet discharge | cfs |
| --- | --- |

(Must be non-erosive)

GENERAL MDC 5: EXCESS FLOW

| Emergency outlet elevation |  | ft |
| --- | --- | --- |
| Emergency spillway width |  | ft |
| Emergency spillway side slopes |  | : 1 |
| Emergency spillway slope |  | % |

GENERAL MDC 6: DEWATERING

| Dewatering method |  |  |
| --- | --- | --- |
| Drawdown orifice size |  | in |

(If applicable)

GENERAL MDC 7: MAINTENANCE ACCESS

| Maintenance access width |  | ft |
| --- | --- | --- |
| Side slopes within maintenance access |  | : 1 |
| Access extend to public right of way |  | Y / N |

(Minimum width of 10 feet)

(Maximum 3:1)

- DRY POND MINIMUM DESIGN CRITERIA

SAND FILTER MDC 1: SEPARATION FROM THE SHWT

| SHWT elevation |  | ft |
| --- | --- | --- |
| Bottom elevation of sand filter |  | ft |

(Determined through soil tests)

(Minimum 1 foot above SHWT for closed systems, 2 feet for open systems)

SAND FILTER MDC 2: TWO CHAMBER SYSTEM

| Separate sediment and sand chambers? | Y / N |
| --- | --- |

(Storage volume in each shall be equivalent)

SAND FILTER MDC 3: SEDIMENT/SAND CHAMBER SIZING

| Sediment chamber volume | ft3 |
| --- | --- |
| Sand chamber volume | ft3 |
| Treatment volume | ft3 |
| Total design volume | ft3 |
| Bypass device elevation | ft |

(0.75 time the treatment volume)

SAND FILTER MDC 4: MAXIMUM PONDING DEPTH

| Ponding depth from top of sand to bypass |  | ft |
| --- | --- | --- |

(Maximum 6 feet)

SAND FILTER MDC 5: FLOW DISTRIBUTION

| Type of distribution |  |
| --- | --- |

(Level spreader, pipe distribution, weirs)

SAND FILTER MDC 6: SAND MEDIA SPECIFICATION

| Drawdown time | days |
| --- | --- |

(2-5 day drawdown to permanent pool level)

SAND FILTER MDC 7: MEDIA DEPTH

| ASTM C33 or equivalent specified? | Y / N |
| --- | --- |

SAND FILTER MDC 8: OUTLET

| Filter bed depth | in |
| --- | --- |
| Depth of sand above underdrain | in |

(Minimum 18 inches)

(Minimum 12 inches)

SAND FILTER MDC 9: MAINTENANCE OF MEDIA

| Maintenance plan provided? | Y / N |
| --- | --- |

(Must specifically indicate maintaining min. 2 in/hr drawdown at sand surface)

SAND FILTER MDC 10: CLEAN-OUT PIPES

| Number of underdrain lines |  |
| --- | --- |
| Number of cleanout pipes provided |  |

(Minimum one cleanout pipe at low point of each underdrain line required)

- SAND FILTER DESIGN CHARACTERISTICS

Sediment chamber width			__________ ft

Sediment chamber length			__________ ft

Sediment chamber depth			__________ ft

Wall thickness			__________ ft

Number of weirs			__________ ft

Weir width			__________ ft

10-year storm water surface elevation	__________ ft

Outlet pipe diameter			__________ ft

Top of sand elevation			__________ ft

At BMP

1- Year	               2-year                  10-year	      _____-year

Inflow                            	      _______ cfs         _______ cfs         _______ cfs       _______ cfs

Routed Outflow	      _______ cfs         _______ cfs         _______ cfs       _______ cfs

At Analysis Point(s) that BMP contributes to

1- Year	               2-year                  10-year	      _____-year

Pre-development                              _______ cfs         _______ cfs         _______ cfs       _______ cfs

Post-development w/o detention _______ cfs         _______ cfs         _______ cfs       _______ cfs

With Detention        	       _______ cfs         _______ cfs         _______ cfs       _______ cfs

- REQUIRED ITEMS CHECKLIST

The following checklist outlines design requirements. Initial in the space provided to indicate the following design requirements have been met and supporting documentation is attached.

Applicant’s Initials

__________ a. Riprap outlet protection, if provided, reduces flow to non-erosive velocities (provide calculations).

__________ b. Floatation calculations are provided if a closed system is designed with less than one-foot SHWT separation.

__________ c. The sediment chamber serves as a forebay to the sand chamber.

__________ d. The media in the sand filter shall be cleaned, washed, course masonry sand such as ASTM C33 or the equivalent. Sand particles shall be less than 2 mm average diameter.

__________ e. The sand filter drainage area is less than five acres (recommendation).

__________ f. Safe access is provided to facilitate cleaning and maintenance.

__________ g. A profile view of the sediment chamber and sand chamber are provided. Water surface elevations are shown on the profile.

__________ h. A plan view of the sand filter is provided with dimensions shown.

NOTE: Executed Stormwater Facility Operations and Maintenance Permit Agreement and payment of surety are required prior to Stormwater Permit issuance.